Ubuntu MAAS Test Report

Contents

1	Overview	3

2	Test Result Summary	3

3	Preparing for the MAAS Environment	3

4	Verifying the System Power-On and Power-Off (IPMI Mode)	8

4.1	Verifying the System Power-Off	8

4.2	Verifying the System Power-On	10

5	Querying Drive, CPU, Memory, and Hardware Information	12

6	Deploying the PXE System	14

7	Deploying the Image System	15

Overview

This document describes how to use the Ubuntu MAAS bare-metal management system to test MAAS functions on different types of Huawei hardware.

Test Result Summary

The following table lists the test results.

| Hardware Type MAAS Function | V3 rack server | V5 rack server |
| --- | --- | --- |
| System power-on and power-off (IPMI) | Supported | Supported |
| Drive, CPU, and memory information query | Supported | Supported |
| PXE system deployment | Supported | Supported |
| OS deployment | Supported | Supported |

Preparing for the MAAS Environment

1. Configure the connection to the Internet.

Set the public IP address, subnet mask, gateway, and DNS server to ensure that you can access the Internet. You can run the ping archive.ubuntu.com command to verify the configuration, as shown in the following figures.

2. Download the image I/O from the official website to the MAAS server.

3. If the DHCP does not exist in the network, enable the DHCP function of MAAS.

(1) Choose Subnets > untagged to go to the VLAN configuration page.

(2) Choose Take action > Provide DHCP, and set the rack controller, start IP address, end IP address, and subnet mask of the DCHP function.

4. Set the MAC address for the server to be deployed, set the power control mode, and add the server to MAAS management.

5. Select the PXE deployment I/O and image installation I/O.

Verifying the System Power-On and Power-Off (IPMI Mode)

Verifying the System Power-Off

After setting Power type to IPMI on MAAS, you can power off a server directly.

(1) On the Machines page, choose Take action > Power off to power off a server.

(2) In the displayed drop-down list box, click Power off machine.

(3) Wait a few minutes. The operation is successful when the power status changes to Power off.

Verifying the System Power-On

A server system can be powered on by using MASS only after MAAS is installed on the server.

For a server system where MAAS is installed successfully, choose Take action > Power on.

Querying Drive, CPU, Memory, and Hardware Information

The actual server information is as follows.

| Component | Information |
| --- | --- |
| System vendor | Huawei |
| System product | XH622 V3 (Type1Sku0) |
| System version | V100R003 |
| System serial | 2102311EPY10J3000009 |
| Mainboard vendor | Huawei |
| Mainboard product | BC21HGSA0 |
| Mainboard firmware version | 3.63 |
| Mainboard firmware date | 05/19/2017 |
| CPU model | Intel(R) Xeon(R) CPU E5-2620 v3 |

Deploying the PXE System

1. Access the OS for which the PXE boot mode needs to be deployed, and choose Take action > Commission.

2. After the deployment starts, the Commissioning operation will be performed for all items. After all items are passed, the Hardware tests operation will be performed.

After the deployment is successful, the status changes to Ready.

Deploying the Image System

1. On the Machines page, select the server where the system is to be installed, and choose Deploy in the upper right corner.

2. The system deployment starts. The status changes to Deploying Ubuntu 18.04 LTS.

3. After the deployment is successful, the status changes to the name of the installed system (Ubuntu 18.04 LTS).